- Task 1

- Circuit design

Design a circuit, which will meet the following requirements.

If the input voltage Vin is higher than +1.4 V, the output voltage Vout is about -13.5 V.

If the input voltage Vin is lower than +1.4 V, the output voltage Vout is about +13.5 V.

- Your solution: - The desired circuit is an inverting comparator: It makes a negative output voltage out of a positive input voltage. - Let’s use a commercial operational amplifier (OA). - The device should invert the polarity. - This implies that the input voltage Vin will be connected to the inverting input of the OA. - The output voltages should be about ±13.5 V. This implies that the supply voltages will be ±15 V. - The comparator should compare the input voltage with a constant voltage of +1.4 V. This reference voltage will be connected to the other - non inverting - input of the OA. - We can use two diodes to generate the reference voltage: 1.4 V = 2 x 0.7 V

Task 2

- Crossword puzzle

- Find the terms corresponding to the following definitions and write them in the table.

- Then use the terms to solve the crossword puzzle below.

| - Definition | - Term |
| --- | --- |
| - An electric circuit that compares two signals | - COMPARATOR |
| - An electronic device that increases the voltage, current, or power of a signal | - AMPLIFIER |
| - Greater than zero | - POSITIVE |
| - Less than zero | - NEGATIVE |
| - Not able to move quickly; not fast | - SLOW |
| - Capable of moving at high speed; not slow | - FAST |
| - The point at which the signal leaves the amplifier | - OUTPUT |

Task 2

Circuit analysis

Refer to the circuit.

- Given the fact that the extreme values of the output voltage Vout are ±13.5V complete the table.

| - V+ [V] | - V- [V] | - Vout [V] |
| --- | --- | --- |
| +5.00 | +5.01 | -13.5 |
| -7 | +2 | -13.5 |
| -3.21 | -3.22 | +13.5 |
| +2.65 | -3.25 | +13.5 |
| +0.001 | +0.002 | -13.5 |
| -0.1 | -0.02 | -13.5 |
| +1.32 | +1.31 | +13.5 |

Doplňte průběh výstupního napětí pro komparátor neinvertující, bez hystereze, s operačním zesilovačem napájeným z +/-15V

Doplňte průběh výstupního napětí pro komparátor neinvertující, s hysterezí, s operačním zesilovačem napájeným z +/-15V

- Metodický list

- Nastavení tisku a zobrazení skrytého textu:

- Ve vodorovné nabídce aplikace Word zvolte Soubor.

- V rozbalené svislé nabídce zvolte Možnosti.

- V okně Možnosti aplikace Word zvolte Zobrazení.

- Ve skupině Vždy zobrazit tyto značky ... nechte položku Skrytý text zaškrtnutou stále. Díky tomu vždy na obrazovce uvidíte správná řešení.

- Ve skupině Možnosti tisku položku Tisknout skrytý text

- zaškrtněte při tisku pro učitele (vytiskne se včetně řešení)

- nezaškrtněte při tisku pro žáky (vytiskne se bez řešení)

- Kontrola, jak bude nebo nebude vytisknutý skrytý text:

- Ve vodorovné nabídce aplikace Word zvolte Soubor.

- V rozbalené svislé nabídce zvolte Tisk. V pravé části obrazovky (viz modrá šipka) vidíte náhled souboru tak, jak bude vypadat po vytisknutí.